RFILE 0.0 release notes                              Author : Sandro Fergnani

Dear B2Streamlines,

I wrote rfile mainly because the CAD software I use to plot the templates

of the airfoils for my models, does include a sophisticated programming

language. Too sophisticated and difficult to learn and master. I knew C

and I decided it wasn' time to learn another programming language.

I learn C by myself and, as a result, I'm not

a good programmer, but I enjoy myself. As you will see, no user interface

at all. Rfile  is a DOS program. You can run it in a dos shell of Windows 95

if you want, but it's a DOS program.

What does rfile do ??

Rfile takes a file with airfoil coordinates and produce another ASCII

file that contain:

1) the airfoil described in segments.

2) The coefficient of moment Cm0.

3) The 0 lift angle of the airfoil.

4) the mean line described by points

Obviously the results are not as accurate as the ones you can get from

wind tunnel, but for my purposes they are enough, and, after all, nobody

can forbid to make the calculation more accurate in the future.

Input file requirements

-----------------------

- The airfoil MUST be described with a length of 1 unit.

- The coordinates must describe a curve that begins from trailing edge,

to leading edge describing the upper surface, and then back to

trailing edge describing the lower surface.

- Empty rows are ignored

- Rows starting with the character '*' are treated as comments.

Example: file describing the Eppler E222 airfoil. Dashed lines are used to

mark the data. They are not included in the file.

------------------------------------------------------------------------------

*

* Airfoil Eppler 222 (E222)

*

*

1.00000  0.00000

0.99669  0.00071

0.98722  0.00291

0.97241  0.00645

0.95270  0.01078

0.92812  0.01560

0.89889  0.02092

0.86547  0.02666

0.82830  0.03269

0.78784  0.03883

0.74458  0.04492

0.69901  0.05079

0.65162  0.05630

0.60292  0.06127

0.55342  0.06558

0.50361  0.06908

0.45399  0.07167

0.40503  0.07326

0.35719  0.07378

0.31091  0.07319

0.26662  0.07147

0.22471  0.06864

0.18555  0.06473

0.14947  0.05981

0.11678  0.05395

0.08774  0.04728

0.06258  0.03994

0.04149  0.03210

0.02461  0.02396

0.01203  0.01578

0.00382  0.00792

0.00007  0.00100

0.00199 -0.00470

0.00990 -0.00993

0.02291 -0.01499

0.04083 -0.01953

0.06351 -0.02340

0.09078 -0.02648

0.12243 -0.02872

0.15821 -0.03010

0.19779 -0.03064

0.24083 -0.03035

0.28690 -0.02932

0.33557 -0.02760

0.38634 -0.02531

0.43867 -0.02254

0.49201 -0.01942

0.54578 -0.01608

0.59935 -0.01266

0.65212 -0.00928

0.70344 -0.00609

0.75268 -0.00320

0.79920 -0.00075

0.84238  0.00118

0.88157  0.00249

0.91618  0.00313

0.94560  0.00306

0.96916  0.00234

0.98625  0.00128

0.99656  0.00036

1.00000  0.00000

------------------------------------------------------------------------------

The program try to interpolate the coordinates to get a smooth curve using

splines algorithms that are quite complicated, but it works.

By default the program calculate the airfoil using 100 segments for each

surface of the airfoil (upper and lower).  and the

output (more on output options later) will contain the coordinates of

starting and ending points of the segments. You can increase the number

of segments up to 2048 with a command line switch.  Why segments ??

There is a reason. The calculated segments are a single chain starting

from trailing edge to leading edge and then back to trailing edge.

Imported in my CAD program with little editing I obtain a file that can

be sent to a plotter. I was luck to find an old A3 plotter by Panasonic.

I modified it to use not only pencils, but a steel tool that I took from

my Dremel. So I can cut templates directly to phenolic boards !!!

This is why I use segments.

The program compute the CM0 and 0 lift angle. this is the part that

need more improvement but the first results are not bad. (at least for me).

One of the output options is the mean line, that is described by points.

The sintax of rfile is:

C:\>rfile inp_file [options] outp_file

where:

inp_file   : file that contain the coordinates

outp_file  : file that contain the results (if omitted stdout is taken)

options:

-n number  number of segments to be used to describe the airfoil.

By default is 100 but up to 2048 is supported.

-fu number Scale factor to be applied to the upper surface of the airfoil

Useful to "play" with airfoils to get various versions.

Number must be not negative. If > 1 it will be use as enlarging

factor. If < 1 ad reducing factor.

-fl number the same as -fu, but it's applied to the lower surface of the

airfoil.

-f  number Value to be applied to both upper and lower surface.

-ml        Print the mean line too.

Example of use:

C:\>rfile e222 -n 1000 foobar

Use the file e222 as input. Output go in the file foobar. The airfoil

will be described by 1000 segments.

C:\>rfile e222 -ml foobar

As before, but 100 segments will be used (the default). The output file

will contain the coordinates of the mean line.

C:\>rfile e222 -n 500 -fu 1.23 foobar

500 segments will be used and the upper surface of the airfoil will be

23% bigger.

OUTPUT

------

I'm proud of the output options of the program because are totally designed

by me. Since the main design goas was to obtain "something" that can be

imported in a CAD program,  the output is highly customizable. Rfile use

a template file called rfile.tpl   that contain sections.

A section begins with the character '#'followed by te section name.

The section is ended by the

token "$end". In each section there are some special tokens. Tokens

start with the  character '^'. There is no limit to the rows that

can be present in a section (in theory).

The program substitute the tokens with the output values that it compute.

Actually there are the following sections:

#airf_data

----------

Used to define how to print  the values of CM0 and 0 lift angle. The

two recognized tokens are ^cm0 and ^alfa0.

#print_upper

------------

The program use this section before to print the segments that

describe the upper surfaces. there are no tokens recognized in this

section. I use it to put in the output file some instructions for my

CAD program, for example set the color and/or the input layer.

#print_lower

------------

As before, but the contents of this section are printed in the output

file before to print the segments that describe the lower surface of

the airfoil. Nice place to put instructions for changing the color and

or the input layer.

#after_lower

------------

This section does not contain recognized tokens, and it's printed after

the segments that describe the lower surface.

#segments

---------

This is the most important section. The recognized tokens are:

^x1    --> X value of starting point of the segment

^y1    --> Y value of the starting point of the segment

^x2    --> X value of the ending point of the segment

^y2    --> Y value of the ending point of the segment

For example: Im my CAD program the command that draw a single segment

is "segmento singolo". Such command takes the coordinates of two points

as parameters. In the sintax of my CAD, each couple of coordinates must

be preceded by te character '/'. So, what is the content of my

#segments section ? here we go:

#segments

segmento singolo / ^x1 ^y1 / ^x2 ^y2

$end

Please note that tokens don't need to be isolated. They can be mixed

within other words. For example if the token is bla_bla^x1_bla_bla, the

output could be bla_bla34.33434_bla_bla. Tokens can be repeated in the

same line too. (I don't see how this could be useful but it's free !!).

#mean_line

----------

Rfile use this section to print the points that describe the mean line.

There are two recognized tokens:

^x  and ^y that are the coordinates of the point.

#before_mean_line

-----------------

This section doesn't have any tokens but it's printed before the points

of the mean line.

I think that the template file can be customized for almost any CAD.

The sequence of section processing is :

1) #airf_data   -> rfile print the values of CM0 and 0 lift angle

2) #print_upper -> rfile print this section

3) #segments    -> use this section to print the segments of the upper surface.

4) #print_lower -> rfile print this section

5) #segments -> use this section to print the segments of the lower surface.

6) #after_lower -> rfile print this section

7) #mean line   -> use this tyo print the point of the mean line